ENGR 290 Assignment 1:

Part I – Materials

Due: Wednesday, Oct. 7, 2026

Show all work.

Avogradro’s number = 6.023*1023

- One mole of solid MgO occupies a cube 22.37 mm on a side.  Calculate the density of MgO (in g/cm3).

- Calculate the dimensions of a cube containing 1 mol of solid magnesium.

- What is the angle between the [110] and [111] directions in a cubic system?

- Identify the axial intercept for the (311) plane.

- At 400 ºC, the fraction of aluminum lattice sites vacant is 2.29 *10-5.  Calculate the fraction at 660 ºC (just below the melting point).  Base your calculation on: 0.76 eV = the energy required to create one vacancy in the aluminum structure.   Boltzmann’s constant:    k=8.62*10-5 eV/K.

- A carbon environment (a hydro carbon gas) is used to set the surface carbon content (CS) at 1.0 wt %.  The initial carbon content of the steel (CO) at 0.2 wt %.  Using the error function table, calculate how long it would take at 1000 ºC to reach a carbon content of 0.6 wt % [i.e. (C- CO) / (CS - CO) =0.5] at a distance of 1 mm from the surface.   D1000°C = 2.98*10-11 m2/s.