Chapter 1, Problem 1.

How many coulombs are represented by these amounts of electrons?

(a) 		(b)

(c) 			(d)

Solution:

- q = 6.482x1017 x [-1.602x10-19 C] = –103.84 mC

(b) q = 1. 24x1018 x [-1.602x10-19 C] = –198.65 mC

(c) q = 2.46x1019 x [-1.602x10-19 C] = –3.941 C

(d) q = 1.628x1020 x [-1.602x10-19 C] = –26.08 C

Chapter 1, Problem 2.

Determine the current flowing through an element if the charge flow is given by

(a) q(t) = (3t + 8) mC

(b) q(t) = (8t2 + 4t – 2) C

(c) q(t) = (3e–t – 5e–2t) nC

(d) q(t) = 10sin(120πt) pC

(e) q(t) = 20e–4t cos(50t) µC

Solution:

Derivative Rules: (f ± g)' = f ' ± g'  and  (a·f)' = a·f '  and  (f·g)' = f '·g + f·g'

  - i = dq/dt = d/dt (3t + 8) = d/dt(3t) + d/dt (8) = 3+0= 3 mA

  - i = dq/dt = d/dt (8t2 + 4t – 2) = d/dt (8t2) +d/dt (4t) + d/dt (– 2) = 16t+4+0= (16t + 4) A

  - i = dq/dt = d/dt (3e–t – 5e–2t) = d/dt (3e–t)- d/dt (5e–2t) = (-3e-t)- (-10e-2t) = (-3e-t + 10e-2t) nA

  - i=dq/dt = d/dt (10sin(120πt)) = d/dt [ 10sin(120πt)] =10 d/dt (sin (120 πt)) =10cos (120 πt). d/dt (120 πt) =10cos (120 πt) (120π) = pA

  - i =dq/dt = d/dt (20e–4t cos(50t)) =

(e ) Steps:

i =dq/dt = d/dt (20e–4t cos(50t)) = 20(-4e^(-4t)cos(50t) - 50e^(-4t)sin(50t))

d/dt[20e^(-4t)cos(50t)]

Take the constant out: (a·f)' = a·f'

= 20 · d/dt[e^(-4t)cos(50t)]

Apply the Product Rule: (f·g)' = f'·g + f·g'

f = e^(-4t), g = cos(50t)

= 20( d/dt[e^(-4t)]·cos(50t) + d/dt[cos(50t)]·e^(-4t) )

d/dt[e^(-4t)] = -4e^(-4t)

d/dt[cos(50t)] = -sin(50t) · 50

= 20( (-4e^(-4t))cos(50t) + (-sin(50t)·50)e^(-4t) )

Simplify:

= 20(-4e^(-4t)cos(50t) - 50e^(-4t)sin(50t))

Chapter 1, Problem 3.

Find the charge q(t) flowing through a device if the current is:

(a)

(b)

(c)

Solution:

- q(t) = i(t)dt+q(0)=3dt+q0=(3t+1)  C

- q(t)=(2t+5) dt+q(0)=t2+5t+0=(t2+5t)  mC

- q(t)=20 cos 10t+π/6+q(0)=(2sin(10t+π/6)+2)μC

Steps:

(c)∫ 20cos(10t + π/6) dt = 2sin(10t + π/6) + C

Take the constant out: ∫a·f(x)dx = a·∫f(x)dx

= 20 · ∫cos(10t + π/6) dt

Apply u-substitution: u = 10t + π/6

= 20 · ∫cos(u) (1/10) du

Take the constant out again:

= 20 · (1/10) · ∫cos(u) du

Use the common integral: ∫cos(u) du = sin(u)

= 20 · (1/10) sin(u)

Substitute back u = 10t + π/6:

= 20 · (1/10) sin(10t + π/6)

Simplify 20 · (1/10) = 2:

= 2 sin(10t + π/6)

Add a constant of integration:

= 2 sin(10t + π/6) + C

Chapter 1, Problem 4.

A current of 7.4 A flows through a conductor. Calculate how much charge passes through any cross-section of the conductor in 20 seconds.

# Solution:

q = i x t = 7.4 x 20 = 148 C

Chapter 1, Problem 5.

Determine the total charge transferred over the time interval of 0  t  10s when  A.

Solution:

q=idt=01012tdt=t24100=25-0=25 C

Chapter 1, Problem 6.

The charge entering a certain element is shown in Fig. 1.23. Find the current at:

(a) t = 1 ms	 (b) t = 6 ms	(c) t = 10 ms

30

Figure 1.23

For Prob. 1.6.

Solution:

(a) At t = 1ms,  15 A

(b) At t = 6ms,  0 A

(c) At t = 10ms,  –7.5 A

Chapter 1, Problem 7.

The charge flowing in a wire is plotted in Fig. 1.24. Sketch the corresponding current.

Figure 1.24

For Prob. 1.7.

Solution:

which is sketched below:

Chapter 1, Problem 8.

The current flowing past a point in a device is shown in Fig. 1.25. Calculate the total charge through the point.

Figure 1.25

For Prob. 1.8.

Solution:

Chapter 1, Problem 9.

The current through an element is shown in Fig. 1.26. Determine the total charge that passed through the element at:

(a)  t = 1 s	 (b) t = 3 s 	(c) t = 5 s

Figure 1.26

For Prob. 1.9.

Solution:

(a)

(b)

(c)  q=05idt=10+7.5+5+5+2.5=10+10+10=30 C

Chapter 1, Problem 10.

A lightning bolt with 10 kA strikes an object for 15  s. How much charge is deposited on the object?

Solution: q = i x t = (10x103) x (15x10-6) = 150 mC

Chapter 1, Problem 11.

A rechargeable flashlight battery is capable of delivering 90 mA for about 12 h.

How much charge can it release at that rate? If its terminals voltage is 1.5 V, how much energy can the battery deliver?

Solution:

q= i x t = (90 x10-3) x (12 x 60 x 60) = 3.888 kC

E = p x t = i x v x t = qv = 3888 x1.5 = 5.832 kJ

Chapter 1, Problem 15.

The current entering the positive terminal of a device is  and the voltage across the device is .

- Find the charge delivered to the device between t = 0 and t = 2 s.

- Calculate the power absorbed.

- Determine the energy absorbed between t = 0 and t = 3 s.

Solution:

(a)

2.945 mC

(b)  	 V this leads to p(t) = v(t)i(t) =

(-0.12e-2t) (0.006e-2t) = –720e–4t µW

(c)  = –180 µJ

Chapter 1, Problem 17.

Figure 1.28 shows a circuit with five elements. If

p1 = –205 W, p2 = 60 W, p4 = 45 W, and p5 = 30 W,

calculate the power p3 absorbed by element 3.

Figure 1.28

For Prob. 1.17.

Solution:

p = 0   -205 + 60 + 45 + 30 + p3 = 0

p3 = 205 – 135 = 70 W

Thus element 3 receives 70 W.

Chapter 1, Problem 18.

Find the power absorbed by each of the elements in Fig. 1.29.

Figure 1.29

For Prob. 1.18

# Solution:

p1 = 30(-10) = -300 W

p2 = 10(10) = 100 W

p3 = 20(14) = 280 W

p4 = 8(-4) = -32 W

p5 = 12(-4) = -48 W

Chapter 1, Problem 19.

Find I and power absorbed by each element in the network of Fig. 1.30.

Figure 1.30

For Prob. 1.19.

Solution:                          I = 8 –2 = 6 A

Calculating the power absorbed by each element means we need to find vi for each element.

p8 amp source = –8x9 = –72 W

pelement with 9 volts across it = 2x9 = 18 W

pelement with 3 bolts across it = 3x6 = 18 W

p6 volt source = 6x6 = 36 W

One check we can use is that the sum of the power absorbed must equal zero which is what it does.